OCT, 24, 1984

CPMRSX is a RSX - version of the program CPMDEC, used in

RT-11 environment. The original version is included as

CPMDEC.FOR.

So also read CPMDEC.DOC, which is the original description

of this program.

Required: RSX-11M (V3.2 was used) and FORTRAN-compiler.

To get the program to work, edit the files:

CPMRSX.CMD and CPMRSX.TKB

to match your system-configuration. The program was compiled

with F4P V3.0 without errors. F4PLIB is not the default

library in our system. ( See CPMRSX.TKB) With FOR V02.2-5

the compilation gave 3 warnings, which we do not understand,

but the program was working properly.

LUN's:

1       DY:     floppy-disk for input

2       SY:     disk forn output

3       SY:     disk for listing

5       TI:     command I/O

***** restriction 1:

The following commands of CPMDEC are tested and working in

CPMRSX:

1      display CP/M directory

2      print (SPOOL) CP/M directory

3      copy a file from CP/M disk

4      copy all files from CP/M disk

9      quit

All others are not testet, or are not working properly. If

you need those - go to work. ( May be CPMDEC.FOR will be

helpfull, this is the original program.)

##### improvement 1

Using the command 3, you will be able to use '*' as a

wildcard, the same way you are used to it from RSX.

***** restriction 2

Since RT-11 and RSX use different file-formats, this program

changes the format, when ASCII-mode is used.

The different I/O structure of FORTRAN makes it necessary,

to send an additional blank at the beginning of each line.

This blank is necessary to print without loosing the first

character in the line. EDI will show you these blanks.

In case, you want to get rid of this, you may use the

following procedure:

EDI your file, using MACRO of EDI:

EDI file

MACRO 1 F C / //M      ! watch, there are 2 blanks after F

V OFF                  ! switch VERIFY off

M                      ! execute the macro

It may be necessary, to do this several times, getting the

next page with REN.

IF you compile with CPMRSXNEW.FTN, this restriction is lifted

and LIST carriage control files are created. Note that

input files must be fixed record length, internal carriage

control for a RSX to CPM format copy to work. See various

XMODEM packages for converter utilities to do this.

When you print this file, the first character in each line

is not printed, due to the flag in the file-header, that

this file is a FORTRAN-generated file.

To change this, use EDI:

EDI newfile

Change from INPUT-mode to command mode

OP file                ! open secondary file

SS             ! select secondary file

PAG 1          ! read all pages

CLOSES         ! close secondary file

SP             ! select primary file

ED             ! exit EDI

This newfile will be in the correct form!

***** programming style

The work of implementing CPMDEC into RSX was done in a

hurry. By this the neat and clean way of programming, done

by RUSS BAKKE has gone lost. That's a pitty, but there was

not the time, to do cosmetic corrections or anything more,

than we needed.

***** problems ?

In the case, you are running into problems, feel free to

contact us.

The address is:

Dr. Hartmut Kreyling

Institut fuer Kernphysik

der Universitaet Frankfurt am Main

August Euler Strasse 6

6000 Frankfurt/Main 90

Tel.: 069 798 4260

or

069 798 4238