Halftone

Robert B. Goldstein and E. Peli

Eye Research Institute of Retina Foundation

20 Staniford St

Boston, MA 02114

(617) 742-3140

This program takes images that are  in  greyscale  form

(ie,  pixel  values  0-255), and converts to halftone (pixel

values 0,1).  It does it  by  an  error  propagation  method

published  in  a paper by Saghri, Hou and Tescher.  The size

of the output image is only dependant on the characteristics

of  the target device, not on the input image.  It therefore

tries to take fullest advantage of the available  pixels  on

the  output device.  Output can be on a sixel device, a text

only CRT, or even on the system lineprinter.

Some useful things you should know if you want  to  try

to get this program to run:

1.  Adage output device.

We have  an  Adage  3000  image  processor  on  our

system.   Since  most sites do NOT have this hardware, I

have removed all support for output to the adage.  I did

this  because  some  of  that software is proprietary to

Adage Corp, and also to eliminate the Undefined  Globals

linker message.

2.  PIX logical

A logical, PIX, must be defined  to  point  to  the

directory which holds your images

3.  Image Format

Probably most sites will  have  their  own  way  of

storing  images.  The image is pulled into memory by the

routines getimage and imlod2.  Re-write  these  routines

so  it  is  appropriate  to your own image formats.  The

routine imlod2 that is provided is appropriate  to  read

the  image  GIRL  that is included with this submission.

GIRL is an image of the face of a girl.  We store images

with  an  associated header file, with an extension .phd

that gives info to the program about  the  image  (size,

etc).   Images  can be read without the .phd header, but

the operator is queried for the missing info.  The  main

item is the size, which, for GIRL, is 128x128.

Page 2

4.  Compilation and linking instructions

On our system the following command lines work:

$ for halftone

$ for half_subs

$ link halftone,half_subs

You  might  have  to  put  for/check=nobounds,  if  your

Fortran  compiler  has  /check=bound  as  a  default.  I

appoligize for this.  For reasons I wont go into, I have

dummy  dimensioned  the image buffer (buf) as having the

dimension 1 in some routines.

5.  Hooks

Hooks are there for some capabilities that I intend

to   put   in,   but   are  not  available  yet.   These

capabilities are:

a.  Output of the processed image to a file.

b.  Re-input of a processed image to just display it.

6.  Speed

Each point in the image  is  multiplied  by  a  2x3

matrix.   This  is  probably  unnecessary, but I haven't

optimized it for speed yet.  As a result, the program is

sometimes   agonizingly   slow.   Future  versions  will

definitely be faster