Ethernet Utilities					    19-NOV-1989

1) ETHMON - an Ethernet Monitor (V2.0)

This is a screen-based (SMG) utility for displaying Ethernet performance

statistics.

VMS Ethernet device drivers keep counts of bytes and packets sent and

received and of various error conditions, such as collisions. Remote

nodes' Ethernet counters are interrogated by using DEC's Maintenance

Operations Protocol and using the fact that VMS device drivers and DEC

terminal servers respond to the necessary requests for information

(so no server processes are needed).

The topological arrangement of the Ethernet can be supplied to ETHMON,

which can then display information about each segment.

See ETHMON.HLP.

1.1) Privileges

PHY_IO privilege is needed to get full information (via the Ethernet

driver's promiscuous mode) but much useful information is available

without it. Note that even if PHY_IO privilege is enabled, by default

ETHMON doesn't use promiscuous mode.

SHARE privilege is needed to get counters for all the local protocols

(as a channel needs to be assigned). WORLD privilege is needed to convert

PIDs to process names. Lack of WORLD or SHARE doesn't cause any problems.

Prior to $DEVICE_SCAN at VMS V5.2, CMKRNL privilege (if available) was

used to scan for Ethernet devices.

NETMBX is needed to create either the promiscuous-mode Ethernet circuit

or the MOP circuit. Without it ETHMON can only display the other local

protocols.

BYPASS is needed for recording of Ethernet packets (/RECORD) or data

logging (/DATA_LOGGING) of them but only if ETHMON is installed with PHY_IO.

This is to protect confidential data (such as plain-text passwords).

In general ETHMON needn't be installed with any privileges (except for

CMKRNL if pre-V5.2 - though this isn't essential). I install it with

PHY_IO and SHARE and use an ACL to control access.

(If it's installed with PHY_IO it's a good idea to limit access.)

1.2) Displays

The main display ("Info") lists the various displays. The qualifiers used

and the Ethernet environment determine which displays are actually available.

Data - data counters for each node.   (*)

Errors - error counters for each node (80 or 132 columns wide).  (*)

<node>-Protocols - a node's activity by protocols.

<node>-Controller - a node's Ethernet controller.

<node>-Activity - node send/receive activity by protocols.

Node-Activity - per-node Ethernet activity.

Protocol-Activity - per-protocol Ethernet activity.

Segment-Activity - per-segment Ethernet activity.

Data-Sources - statistics on each data source

(node+protocol combination).

One display for each local protocol.

(*) These displays show either the absolute counters, as accumulated

since booting or explicit counter reset, or relative counters,

showing the changes between ETHMON samplings. They also show an

approximate Ethernet transmission bandwidth usage based on total

bytes-sent per sampling-period (/INTERVAL=n).

1.3) Keys

Pressing Enter (or Return or Select) toggles between the Info display

and a statistics display.

Keys while at a statistics display:

Left-arrow: previous display

Right-arrow: next display

Gold-left: first statistics display

Gold-right: last statistics display

Up-arrow: scroll up 1 line

Down-arrow: scroll down 1 line

Prev-screen: scroll up 2/3 screen

Next-screen: scroll down 2/3 screen

Gold-prev-screen: scroll to top

Gold-next-screen: scroll to bottom

PF3: toggle between brief & full sets of information

Gold-PF3: toggle between reltive & absolute values of counters

Keys while at the Info display:

Arrow-keys: move around the displays.

Gold-up-arrow: top of column

Gold-down-arrow: bottom of column

Gold-left-arrow: first column

Gold-right-arrow: last column

Prev-screen: scroll up 2/3 screen

Next-screen: scroll down 2/3 screen

Gold-prev-screen: scroll to top

Gold-next-screen: scroll to bottom

Other keys:

Ctrl/W: redraw screen

Ctrl/L: toggle data logging on/off

Ctrl/N: reset the local node's Ethernet controller counters

Gold-N: set display width to 80

Gold-W: set display width to 132

1.4) Statistics

Notes on statistics - see also the NCP Manual (Appendix A) and the

DECserver 100/200/500 Management Guide.

Bandwidth: percentage of the theoretical 10 megabits per second

bandwidth used.

Byte rate: the average number of data bytes transmitted per second.

Collision check failed: the collision check (heartbeat)

following a transmission failed.

Data overrun: the hardware missed a packet.

Receive failed: various reasons.

Received CRC error.

Received framing error - not a multiple of 8 bits.

Received frame too long.

Send deferred: packet sent after being deferred as the

Ethernet was busy. Should be less than 20% of frames sent.

Send 1 Collision: packet sent after 1 collision.

Should be less than 5% of frames sent.

Send 2+ Collision: packet sent after 2 or more collisions.

Should be less than 5% of frames sent.

Send failed: various reasons.

Should be very rare.

Excessive collisions (16+).

Carrier check failed - loss of Carrier signal.

Short circuit.

Open circuit.

Transmitted frame too long.

Remote failure to defer.

Heartbeat error.

Data underflow.

System buffer unavailable: the driver missed a packet.

Unknown/unrecognised frame destination: an unwanted packet was received.

The hardware picked up a packet but the driver couldn't find

a protocol that wanted it.

User buffer unavailable: a protocol missed a packet.

1.5) Data logging

Data from the Ethernet can be logged (/DATA_LOGGING) to a file and then

formatted (/DUMP). If ETHMON is installed with PHY_IO, BYPASS privilege

is needed to use this.

Log files contain variable length records in this format:

header size - 2 bytes

{  system time - 8 bytes

{  $QIO IO$_READVBLK P5 buffer - 20 bytes

data - rest of the record

1.6) Known problems/restrictions

A lot of effort has gone into making ETHMON's data collection as fast

as possible but more can be done. At the moment, any fairly busy

Ethernet has too much data for promiscuous mode to capture and packets

are missed. ETHMON shuts promiscuous mode down automatically if the

packet-loss rate gets too high (see /CLOGGED).

The interval between each polling of remote nodes via MOP is the main

scan interval (/INTERVAL) divided by the number of known

MOP-responding nodes, except that there are at least 10 centisecs

between successive polling transmissions. Thus if there are not

too many nodes, fresh information should be available for each node in

each scan interval. For the default /INTERVAL=4, the critical

number of nodes is (4x100cs/10cs) = 40.

Support for more than one Ethernet controller on a VAX is limited.

Essentially ETHMON will only work fully when using DECnet's

controller and any protocols on other controllers will appear

as if on the DECnet controller. /CONTROLLER=device-name specifies

an alternate controller.

If the local controller counters are reset while ETHMON is running,

the controller counter changes shown by ETHMON sometimes become

huge - I suspect this is a subtle timing problem.

The playback feature is new and hasn't been tested thoroughly yet.

It occasionally gets counters wrong (some for of data corruption?).

Again, this may be a timing problem.

If $NICONFIG is running it seems that the device driver may not supply

information to other nodes upon request (and $NICONFIG won't do it either).

While TSCON is connected to a remote console, not all the usual

information is supplied by the terminal server (this is a restriction

in the terminal server).

There is limited support for IEEE 802.2/802.3 packets.

Various displays may show the SCA (& other) protocol's bytes-sent or

bytes-received count as zero. This happens when the internal counter

has reached MAXINT (%XFFFFFFFF) and is no longer being incremented.

SCA (if present at all) is usually the protocol with highest

bytes-sent or bytes-received value.

DECserver 100s (software versions V1.3 & V2.0) don't supply the

seconds-since-last-zeroed statistic properly. They supply its value

at the time of the most recent server SHOW COUNTERS command.

Accordingly ETHMON shows this value as "> n".

I have found that XEDRIVER (for DEUNAs and DELUAs) does not always respond

to MOP requests properly - $NICONFIG has trouble with it as well.

1.7) Future plans

Experimenting with a separate process for data collection.

Extending the use of knowledge of the Ethernet topology.

Handling multiple Ethernet controllers on the local VAX properly.

Background monitoring, event & summary reporting.

1.8) Note

There is a known bug in all VMS Ethernet drivers from V4.4 to

V5.0-2 (at least) which is avoided by ETHMON. If promiscuous mode is

started with driver data padding enabled, it will eventually crash

the system.

2) TSCON - a terminal server utility.

This communicates with the remote console port of DEC terminal servers

using MOP, allowing remote management of terminal servers.

It can be used like SET HOST for an interactive session, or can be

used to execute a script from a file, or a single command from the

command line.

See TSCON.HLP.

It needs a DECnet Ethernet circuit to be running and for the

terminal server to be registered in the volatile database.

TSCON's interactive mode is slightly different if used on a SET HOST

(RTAn) terminal as the driver does not implement all of TTDRIVER's features.

3) Glossary (NOT comprehensive!).

Circuit: see port.

Frame: see packet.

MOP: the Maintenance Operations Protocol, used for downloading

software, NCP's CONFIGURATOR subsystem ($NICONFIG),

terminal server management (TSCON, NCP CONNECT, TSM) and other

management tasks.

Multicast: a "broadcast" address as opposed to a specific node address.

E.g. DECnet-routers, LAT-hosts, VAXcluster group-number.

$NICONFIG: the process run by NCP upon doing NCP SET MODULE

CONFIGURATOR KNOWN CIRCUIT SURVEILLANCE ENABLED.

It scans the Ethernet using MOP to find other nodes.

Packet: an Ethernet packet contains between 46 and 1500 bytes of

data plus a header of source-address, destination-address,

protocol and CRC. The 4-byte CRC is often not counted in

statistics about bytes transferred (as it's usually generated

and checked by the Ethernet controller hardware).

Port: an instance of the use of a protocol.

For shared protocols there can be several ports per protocol.

The IO$M_STARTUP $QIO modifier is used to create a port.

Promiscuous mode: this makes the Ethernet controller collect every packet

on the Ethernet, rather than filtering out the ones not wanted by

the local node.

Protocol: a label usually corresponding to a software subsystem,

e.g. DECnet, dump/load, TCP/IP, loopback, MOP.

An Ethernet protocol is represented in VMS by a "cloned" device.

E.g. for a DEQNA, the template device is usually XQA0 and the devices

are show by SHOW DEVICES XQA. Each protocol can be exclusively-owned,

with the device non-shareable, or can be shared, in which case the

device is marked shareable. Each shared protocol can have one default

user and any number of shared-with-destination users.

SCA: is the System Communication Architecture (VAXcluster comms).

TSM: Terminal Server Manager, DEC's software tool for managing

terminal servers.

4) References.

DECnet DNA Phase IV Ethernet Data Link Functional Specification

AA-Y298A-TK  December 1983

DECnet DNA Phase IV Maintenance Operations Functional Specification

AA-X436A-TK  December 1983

DECserver 100 Management Guide

AA-KY60A-TK  June 1988

(& presumably DECserver 200 & DECserver 500 equivalents).

VMS I/O Users Reference Manual: Part II

AA-LA85A-TE  April 1988

VMS Network Control Program Manual

AA-LA50A-TE  April 1988

Ian Kitching,

System Manager,

Computing Service,

Anglia Higher Education College,

East Road,

Cambridge CB1 1PT    Tel. 0223 63271 ext. 2189