LASER is a user-written print symbiont that is tailored specifically to the

Apple LaserWriter family, but works with other PostScript printers,

including DEC printers.

The symbiont as distributed is configured to print normal text files in

a variety of formats on the laser printer.  Support is also included for

printing PostScript, MacPaint, Tektronix plot, and Zeta plot files.  See

LASER.HLP and LASER.C for additional capabilities of the symbiont.

This distribution consists of the following files:

File		Destination		Purpose

---------------------------------------------------------------------------

AAAREADME.DOC				This file

LASER.EXE	SYS$SYSTEM:LASER.EXE	The symbiont executable.

*.PS		SYS$LIBRARY:LASER.TLB	Library of PostScript header files.

LASER.C					Symbiont sources.

LASER_ERR.MSG				Error message source.

QUIDEF.H				Source header files.

SMBDEF.H

LBRDEF.H

LASER.OPT				Link options file.

LASER.HLP	SYS$HELP:HELPLIB.HLB	Description of forms supported.

SYST.COM	SYS$MANAGER:SYSTARTUP.COM Queue startup commands.

Installation.

Installation of the files can be accomplished with the following set of

commands:

$ LIBRARY/CREATE/TEXT LASER *.PS

$ COPY LASER.EXE SYS$SYSTEM:/PROT=W:RE

$ COPY LASER.TLB SYS$LIBRARY:/PROT=W:RE

$ LIBRARY /HELP SYS$HELP:HELPLIB LASER		! Optional

Select a terminal port to attach the printer to, and edit SYST.COM to

reflect your change.  SYST.COM may be invoked by your system startup

procedures.  The printer may be connected with a cable that has only

transmit, receive and ground pins connected.  Transmit and receive

should (I think) be swapped. The switch on the LaserWriter should be

switched to the "9600" position.  If you are sharing your LaserWriter

with a Macintosh connected via AppleTalk, use the following procedure

when switching the printer:

NOTE: If your printer is connected via a LAT device, the name of the device

that you specify in your START/QUEUE command must have letters LTA in it.

The symbiont checks the name and searches for LTA.  If it finds it, the

software deassigns the device between print jobs to allow sharing of the

printer with more than one node.

To switch the printer to the VAX:

1. Insure that no AppleTalk jobs are in progress.

2. Disconnect the appletalk connector from the laserwriter.

3. Switch switch on LaserWriter to "9600".

4. $START/QUEUE ALW

To switch the printer to the MAC:

1. $STOP/QUEUE/NEXT ALW

2. Allow any jobs in progress to complete.

3. Switch switch on LaserWriter to "AppleTalk".

4. Connect the appletalk connector to the laserwriter.

To build the symbiont from the sources, use BUILD.COM.  Edit it first and

enable definitions that are described in it to suit your site.

PostScript is a trademark of Adobe Inc.

LaserWriter and MacPaint are trademarks of Apple Computer.